NEGISHI LABS CERTIFICATION OF HAZARD ASSESSMENT	p. 1 of 1

| Supervisor (print): Prof. E. Negishi | Assessment Date(s): May 23, 2003 |
| --- | --- |
| *Signature: | Location(s) posted: WTHR 454, WTHR 465 |

| Hazards | Task: | Minimum Requirements |
| --- | --- | --- |
| Skin/eye damage from impact or splash, tissue damage from extreme cold, poisoning, inhalation of vapor or aerosol | Handling or standing within 2 ft of any volume < 150 mL of organic liquid, liquid mixture, corrosive (pH < 2 or pH >10) liquid, or any toxic liquid or liquid mixture. Handling dry ice or liquid nitrogen | Safety glasses, chemical resistant gloves, skin cover to knees/elbows/throat, closed shoes with socks. All vaporous chemical work to be done in hood. Shower and eyewash must be available in work area. |
|  | 150 mL ≤ volume < 5 L | as above with increased protection via lab coat over clothes, chemical splash goggles and cover to ankles/wrists/throat |
|  | volume > 5 L | add face shield covering chin |
| Skin/eye damage | volume ≤ 1 L hot liquid (water bath, oil bath, autoclavel...) | Safety glasses, skin cover to knees/elbows/throat, closed shoe w/ socks |
|  | volume > 1 L | skin cover to throat/wrists/ankles, emergency shower available in work area |
| Skin/eye damage, poisoning, inhalation of airborne dust | ≤ 100 g any hazardous solids or ≤ 1 g "chemical requiring designated area," | Safety glasses, goggles for large quantities, chemical resistant gloves, skin cover to elbows/knees/throat, closed shoes/socks |
|  | > 100 g any hazardous solid, or > 1 g "chemical requiring designated area," (list at REM web site) or high potential for airborne particles | same, except add/change to chemical splash goggles, skin cover to wrists/ankles, and only work in hood |

* Signature of supervisor above affirms that rules have been designed to fit the work and work area and are the supervisors best assessment of how to prevent injury and illness.  If any new work or new PPE not covered above are undertaken, the requirements must be reassessed immediately.  Supervisor signature also affirms the intent of the supervisor to (1) enforce the PPE rules, (2) discipline any group members who fail to observe them, and (3) document and/all disciplinary actions and report them to the Safety Committee immediately.

The electronic version of this Word document is file is available at

http://www.chem.purdue.edu/chemsafety/SOPs/GroupHazAssCerts.htm

Professor Negishi may modify at any time to improve the provisions of the document, and must modify at any time new hazards are introduced or new PPE needs arise for any reason.  Group members are encouraged to participate in the process of suggesting improvements to the group PPE policies.