2005 Chemistry Self-audit Checklist page 1 of 2

date:______	bldg/room(s):__________	group ___________	inspected by ______

Due April 30.  All “not OK” items to be corrected before turning in checklist(s), or attach a clear explanation and estimate the date of correction.  Keep group checklists together and staple for submitting.  You may group some rooms together on a single checklist page, or use multiple checklists, as you find convenient and appropriate, but use different pages for different dates.

| Correct * items immediately if found not OK. |  |  | OK, not OK, or N/A | Additional explanation. |
| --- | --- | --- | --- | --- |
|  |  | No chemicals (or chem waste) storage in walking/working floor space. |  | Clearly marked floor storage areas are allowed if unused aisle remains > 3ft wide. |
|  |  | All flammable liquids except 10 gal are stored in fire protected containers (flamm cabs or safety cans). |  | Flash point <60C. (140F) OK to have up to 10 gal outside of fire protected containers (in closed containers). INCLUDES WASTE. |
| * |  | Gas cylinders all supported correctly |  | Approved chain/strap, solid attachment, no grouping of any un-capped cylinders. |
|  |  | Hood is used for work OR for chemical storage, not both |  | A few bottles for a few days does not constitute “storage.” |
|  |  | Hood is used correctly. Check face velocity, no perchloric acid use. |  | Includes overcrowding, too much material in front 6 inches, and obstruction of sash. |
|  |  | Lab door to public hallway is posted with current emergency contact information. |  | 24 hour contact person name, who can answer detailed questions. |
|  |  | Waste accumulation is < 30 gal |  | > 30 gal is easy to spot. |
|  |  | Hazardous waste containers are labeled adequately. |  | Adequate = the word "waste" + chemical name(s) or description. |
|  |  | Waste container(s) are all closed securely. |  | Loose screw cap is same as "open," but "Eco-funnel" is OK. |
|  |  | All containers labeled with readable contents name or list. |  | Acronyms or formulae are only OK if there is a "key" posted and it is easy & effective. |
| * |  | Walking and working floor area free of trip/slip hazards, there is easy path to all exits and eyewash/shower. |  | Require: 2 ft inner aisles, 3 ft aisles leading to door if original construction permits. (Also no cords, hoses on floor.) |
|  |  | No spill of solid(s) or liquid(s) on floors, equipment, or working surfaces. |  | Spill of solid(s) or liquid(s) on floors, equipment, or working surfaces. |
| * |  | Eyewash and/or shower unobstructed. |  | Stuff in it or in front of it, over it…. |
|  |  | Regular trash, bio waste, or glass/sharps disposal area is not overcrowded. |  | An indicator of bad habits, and a predictor of bad inspection results. |
| * |  | No “unstable storage” or assembly which could fall or person could run into it |  | Stacked up boxes, sticking-out items, hanging cords, tubing... |
| * |  | All doors open freely, not blocked by furniture or equipment. |  | Most are required emergency exits from somewhere, often the other room. |
|  |  | Liquids all stored at or below eye level. |  | Must move liquids down or must write SOP for accessing liquids at heights. |
|  |  | Drill press, pumps, motors, saws, & grinders have guard over pulley, blade... |  | Older vacuum pumps, or hydrogenator shaker, grinder... in labs and/or shops.….. |
|  |  | Plenum above dropped ceiling is free of flammable materials. |  | It is not acceptable to run non-approved cords, data wire, hoses, etc., up there. |
|  |  | All power cords checked, no damage. |  | Tape of any kind is NOT an acceptable method of repairing cord insulation. |

Make comments on separate page, if necessary, and attach.  Write item number from this page next to a comment to clearly associate with a particular item.

p 1 of 2

http://www.chem.purdue.edu/chemsafety/LimAcc/2005ChemistrySelfAuditChecklist.doc

2005 Chemistry Self-audit Checklist page 2 of 2

date:______	bldg/room(s):__________	group ___________	inspected by ______

Due April 30.  All “not OK” items to be corrected before turning in checklist(s), or attach a clear explanation and estimate the date of correction.  Keep group checklists together and staple for submitting.  You may group some rooms together on a single checklist page, or use multiple checklists, as you find convenient and appropriate, but use different pages for different dates.

| Correct * items immediately if found not OK. |  |  | OK, not OK, or N/A | Additional explanation. |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Hazard assessment is posted prominently and is appropriate to work in area. |  | To contain necessary PPE only, not a copy of the template. |  |
| * |  | All fire extinguishers, sprinklers, alarms, electrical service panels unobstructed. |  | Can you see it? Get to it fast? (Sprinkler requires 18" free space hemisph.) |  |
| * |  | Appropriate PPE in use or and used correctly. |  | Must note rules in Hazard Assessment and watch PPE behavior to ascertain this. |  |
|  |  | All glass apparatus subjected to vacuum or pressure is caged or taped. |  | Dewars, vacuum line, desiccators, vacuum manifold. |  |
| * |  | Incompatible chemicals are stored separately. |  | Most often violation is acid + base or oxidizer + flammable. |  |
|  |  | Radioactive materials are kept in secure area. |  | If no people are around, the room or the isotopes/source must be locked up. |  |
|  |  | All toxic gases are kept in a continuously ventilated enclosure. |  | See gas list at http://www.chem.purdue.edu/ chemsafety/Chem/PoisonGases.doc |  |
|  |  | Class 3 b or 4 laser area has access controls and/or clear warnings. |  | Curtains, warning lights, signs…. |  |
|  |  | Rad items are stored/used only in permitted areas. |  | Rad symbol sticker on outside of door IDs room as rad use room. |  |
|  |  | Eating/drinking policies are known and carefully observed in rad use rooms. |  | Rad door sticker states what is not allowed. Evidence sometimes obvious. |  |
|  |  | Class 3 b or 4 lasers used only by trained individual(s) |  | Check laser training documentation. |  |
|  |  | Class 2 or higher biohazards used only inside of proper containment. |  | Ask about biohaz class for all bio work |  |
|  |  | Extension cords used temporarily or not at all. (A fused power strip is OK.) |  | Temporary means a PART DAY, not multiple days. |  |
|  |  | Bloodborne pathogens used only by trained individual(s). |  | Check training records. Annual refresher training required for BBP. |  |
|  |  | BBP used only by individual(s) who have been vaccinated or declined in writing. |  | This should be documented in the group training records. |  |
|  |  | Rad materials used only by trained individual(s). |  | Check training records. Annual refresher training required for rad user |  |
|  |  | CHP training complete where appropriate. |  | Check training records. CHP refresher required after 5 years. |  |
|  |  | PPE training complete where appropriate. |  | PPE refresher not required unless required by supervisor. |  |
|  |  | HazCom training complete where appropriate. |  | Also called HCP or Right-to-Know. For non-lab people. |  |
|  |  | Staff members are aware of and familiar with safety manual locations and contents (CHP, Haz Com, Laser, Biosafety, Rad Safety, as appropriate). |  | CHP manual is appropriate in labs, HazCom manual in non-lab areas where "chemicals" are used/stored (“shops”). Biosafety & Rad are variable. |  |

Make comments on separate page, if necessary, and attach.  Write item number from this page next to a comment to clearly associate with a particular item.

p 2 of 2

http://www.chem.purdue.edu/chemsafety/LimAcc/2005ChemistrySelfAuditChecklist.doc