# Approved flamm storage = safety cans and flamm cabs.

# They must be working properly and cannot be cut into.  Cabinets are not vented except by special permission of Fire Safety.

There may be NOT MORE THAN 10 gallons combined Class I plus Class II liquids OUTSIDE of approved flammable storage in a "fire area."  (A fire area is separated from other areas by fire doors.  We usually use "room" as the unit, rather than fire area.)

Doors are to be self-closing and equipped with a latch

COMBINED CLASS I AND CLASS II LIQUIDS:

Flash point <38 oC and bp < 60 oC.  EXAMPLES

1-butanol

1-propanol

2-butanol

2-propanol

acetic acid

acetone

acetonitrile

benzene

diethyl ether

dimethylformamide

ethanol

ethyl acetate

heptane

hexane

isopropyl alcohol

methanol

methyl ethyl ketone

n-heptane

n-hexane

n-nonane

n-octane

n-pentane

p-dioxane

pentane

propionic acid

propylene oxide

tetrahydrofuran

toluene

xylenes